Laboratory IT110 Concordia University Instructor: Bernal Manzanilla Saavedra

Lab Activity: The Latency Race (Registers vs. RAM)

IMPORTANT: Do NOT submit written answers for this lab. This is a self-study guide to prepare you for the Moodle Quiz.

Background: Access the emulator page: https://www.peterhigginson.co.uk/LMC

Inside a computer, the Accumulator (a CPU register) is incredibly fast because it is built directly into the processor. The RAM (the Mailboxes) is much slower because data has to travel back and forth across a physical wire called a bus.

Today, you will test two programs that do the exact same thing: Ask for 3 numbers, add 1 to each, and output the results. However, they are written differently.

Program A: The "RAM Hog" This program asks for all 3 numbers first, saves them all to RAM, and then does the math.

- Type this into the Assembly box, Assemble, and Run it.

- Count how many total steps (fetches/executes) it takes.

PROGRAM A

INP

STO 90      // Write to RAM

INP

STO 91      // Write to RAM

INP

STO 92      // Write to RAM

LDA 90      // Read from RAM

ADD 99

OUT

LDA 91      // Read from RAM

ADD 99

OUT

LDA 92      // Read from RAM

ADD 99

OUT

HLT

99 DAT 001  // Constant 1

PROGRAM B

INP

ADD 99      // Math done directly in Accumulator

OUT

INP

ADD 99

OUT

INP

ADD 99

OUT

HLT

99 DAT 001  // Constant 1

Self-Study Questions:

- Self-Check Q: How many extra instructions did Program A have to run compared to Program B?

- Self-Check Q: Notice how Program A constantly uses STO and LDA. If traveling to a Mailbox takes 10 times longer than doing math in the CPU, how does this affect the computer's latency?

- Self-Check Q: Why is it better to keep data in the Accumulator if you are going to use it immediately?