Laboratory IT110 Concordia University Instructor: Bernal Manzanilla Saavedra

Lab Activity: Bit Shifting and Data Masking

IMPORTANT: Do NOT submit written answers for this lab. This is a self-study guide to prepare you for the Moodle Quiz.

Background: At the hardware level, multiplying and dividing numbers takes several clock cycles. However, CPUs can shift bits to the left or right in a single clock cycle. By manipulating bits directly, programmers can optimize code for extreme speed and pack data tightly to save space.

Activity 1: Logical Shifts (Speed Optimization) A Logical Shift Left (LSL) moves all bits to the left, adding a 0 to the rightmost empty spot. A Logical Shift Right (LSR) moves all bits to the right, adding a 0 to the leftmost spot.

- The Math Rule: Shifting left by 1 bit multiplies the number by 2. Shifting right by 1 bit divides the number by 2 (dropping remainders).

Exercise: Take the 8-bit unsigned binary number 0000 0101 (Decimal 5).

- Shift it left by 1 bit. What is the new binary number and its decimal value?

- Shift the original number left by 3 bits. What is the new decimal value?

- Take the binary number 0001 1000 (Decimal 24). Shift it right by 2 bits. What is the new value?

- Self-Check Answers:

  - 1: 0000 1010 (Decimal 10. 5×2=10).

  - 2: 0010 1000 (Decimal 40. 5×23=40).

  - 3: 0000 0110 (Decimal 6. 24/22=6).

Activity 2: Arithmetic Shifts (Sign Preservation) If a number is signed (Two's Complement), a standard right shift would ruin the sign bit. An Arithmetic Shift Right (ASR) shifts the bits but copies the original sign bit into the new leftmost spot.

Exercise: Take the 8-bit signed negative number 1111 1100 (Decimal -4).

- Perform a Logical Shift Right by 1 bit. What happens?

- Perform an Arithmetic Shift Right by 1 bit. What happens?

- Self-Check Answers:

  - 1 (LSR): 0111 1110. The number becomes a massive positive integer. The sign was destroyed!

  - 2 (ASR): 1111 1110. The 1 is preserved. The value becomes -2. The math works!

Activity 3: Bit Masking (Space Optimization) To save memory, computers often pack multiple data points into a single 8-bit register. To read just one piece of that data without being confused by the rest, we use a Bit Mask with the logical AND operator.

- 1 AND 1 = 1

- Anything AND 0 = 0 (This "masks" or hides the bits we don't care about).

Exercise: You have a register containing 1011 0110. You only care about the lower 4 bits (the right half).

- Create a mask that isolates the lower 4 bits. (Hint: put 1s where you want to keep data, 0s where you want to erase it).

- Apply the AND operator between the register and your mask.

- Self-Check Answers:

  - Mask: 0000 1111

  - Result: 0000 0110. The upper bits were successfully erased, leaving only the data you wanted!