Lab 3 IT 213

Bernal Manzanilla Saavedra January 30th, 2026

### Handout 1: The "No-Install" UDP Challenge

(For All Students)

Title: Lab 3: The "Fire and Forget" Protocol

Objective: Observe UDP traffic behavior using standard Windows tools.

IMPORTANT:

Gather in groups! Find a partner who has Wireshark installed. You will do this activity together: one person checks the "Scoreboard" (this handout), while the other watches the "Live Game" (Handout 2).

#### Part 1: The Baseline (The Scoreboard)

Before we start, we need to see how quiet the network is.

- Close Everything: Close all web browsers, Spotify, Discord, Steam, etc. We want your network to be as silent as possible.

- Open Command Prompt: Press the Windows Key, type cmd, and press Enter.

- Check the Score: Type the following command to see your total UDP statistics:

- DOS

- netstat -s -p udp

- (Hint: -s means statistics, -p udp means "show me only UDP protocol".)

- Record the Number: Look for the line "Datagrams Received".

  - Starting Count: ________________

#### Part 2: Generating the Traffic

Now we are going to force your computer to use UDP.

- Action A (The Lookup):

- In your Command Prompt, type: nslookup google.com

  - Do this 5 times quickly.

  - (Hint: nslookup asks a DNS server for an IP address. DNS uses UDP because it needs to be fast—like shouting a question across a room.)

- Action B (The Firehose):

- Open your web browser (Chrome/Edge) and go to YouTube. Search for a "4K Nature Video."

  - Set the quality to 4K (2160p).

  - Let it play for 30 seconds.

  - (Hint: Modern streaming uses a protocol called QUIC. QUIC runs on top of UDP to send video data faster than TCP ever could.)

#### Part 3: The Aftermath

- Go back to your Command Prompt.

- Run the stats command again:

- DOS

- netstat -s -p udp

- Record the New Number: ________________

- Calculate the Difference: New - Old = ________________ UDP Packets.

Critical Thinking Question:

Why did watching a video generate thousands of UDP packets? If a few of those packets (pixels) got lost, would the video stop and wait for them?

#### Part 4: Be the Computer (Manual Decode)

Since we cannot see the headers in Wireshark on every laptop, you will decode one manually.

Below is a Real UDP Header captured from a gaming session (in Hexadecimal code).

Hex Dump: 04 d2 00 35 00 1c a9 f1

Use this Reference Table to decode it:

| UDP Header Field | Size | Position in Hex | Meaning |

| :--- | :--- | :--- | :--- |

| Source Port | 2 Bytes | 1st & 2nd pair | The sender's "Door Number" |

| Dest Port | 2 Bytes | 3rd & 4th pair | The receiver's "Door Number" |

| Length | 2 Bytes | 5th & 6th pair | Total size of packet (Header + Data) |

| Checksum | 2 Bytes | 7th & 8th pair | Error checking code |

Your Task:

- Identify the Destination Port (3rd & 4th pair): 00 35

  - Convert 0035 from Hex to Decimal (Use your phone or Windows Calculator in "Programmer" mode).

  - Decimal Port: _______

  - What Service is this? (Hint: It’s the "Phonebook" of the internet). ______________

- Identify the Length (5th & 6th pair): 00 1c

  - Convert 001c from Hex to Decimal.

  - Total Bytes: _______

### Handout 2: The Wireshark Supplement

(For Students WITH Wireshark)

Title: Lab 3: Seeing the Invisible

Objective: Verify the "No-Install" findings using actual packet capture.

IMPORTANT:

Work with your group! While your partners run the commands in Handout 1, YOU are the "X-Ray Machine." You will tell them what is actually happening on the wire.

#### Activity 1: Capturing the DNS Lookup

Your partner is running nslookup. Let's see the packet leave.

- Open Wireshark: Double-click your active network card (usually "Wi-Fi" or "Ethernet").

- Set the Filter:

  - In the bar at the top, type: dns (lowercase).

  - Press Enter. The screen should be blank (this hides all the noise).

- The Trigger: Have your partner run nslookup google.com in their terminal.

- The Capture:

  - You should see two packets appear immediately:

    - Right Arrow ---->: "Standard query A google.com" (The Question).

    - Left Arrow <-----: "Standard query response..." (The Answer).

- The Inspection (Click the packet with the Right Arrow):

  - In the middle pane, find the line User Datagram Protocol and click the arrow > to expand it.

  - Verify for the Group:

    - Does the "Destination Port" say 53?

    - Does the "Length" say 8 bytes? (The UDP header is always 8 bytes!).

#### Activity 2: Capturing the Video Stream (The Flood)

Your partner is about to play a 4K video. Get ready.

- Reset: Click the Red Square (Stop) and then the Blue Shark Fin (Start) to clear the screen.

  - If it asks to save, say "Continue without Saving".

- Set the Filter:

  - Change the filter bar to: udp

  - Press Enter.

- The Trigger: Have your partner start the YouTube video at 4K Quality.

- The Observation:

  - Watch your screen. You should see a "Waterfall" of packets scrolling impossibly fast.

  - Note: Wireshark might label these as QUIC. This is normal! QUIC is a modern version of UDP used by Google/YouTube.

- The Analysis:

  - Click on any random blue/QUIC packet.

  - Expand the User Datagram Protocol section.

  - Question for the Group: Look for an "Acknowledgment Number" or "Sequence Number" (like we saw in TCP).

  - Answer: Do you see one? (Hint: No. UDP doesn't care if packets arrive in order. It just sends them!).